2010 Residential Lighting Report

APPENDIX 4

April 2013

This work is licensed under the Creative Commons Attribution 3.0 Australia Licence. To view a copy of this license, visit http://creativecommons.org/licences/by/3.0/au The Department of Climate Change and Energy Efficiency on behalf of the Equipment Energy Efficiency Program asserts the right to be recognised as author of the original material in the following manner: © Commonwealth of Australia (Department of Climate Change and Energy Efficiency) 2011. The material in this publication is provided for general information only, and on the understanding that the Australian Government is not providing professional advice. Before any action or decision is taken on the basis of this material the reader should obtain appropriate independent professional advice. This document is available at www.energyrating.gov.au While reasonable efforts have been made to ensure that the contents of this publication are factually correct, E3 does not accept responsibility for the accuracy or completeness of the content, and shall not be liable for any loss or damage that may be occasioned directly or indirectly through the use of, or reliance on, the contents of this publication.

Methodology - Field Survey Manuals

Lighting Field Guide – Lamp Fittings

Only consider fittings that are mains powered.

| Fitting | Pictures | Notes |
| --- | --- | --- |
| Batton Holder |  | A bare fitting, normally fixed directly to the ceiling |
| Batton Holder with Shade |  | A bare fitting, normally fixed directly to the ceiling, with a surrounding shade |
| Linear Fluorescent |  | Linear fluorescent, can be suspended, surface mounted or flush mounted. May or may not have a diffuser (cover) |
| Oyster |  | Batton Holder with cover (of any material), ceiling mounted only |
| Suspended |  | Any type of hanging fitting, may have more than one lamp, may be covered (with any material) or uncovered) |
| Downlight/Flush Mounted |  | Recessed and flush with ceiling |
| Indoor Spotlight |  | Not flush mounted, may have a gimble fitting (allowing tilting of the lamp) |
| Uplight | No picture example – these fittings will be uncommon in houses | Directional fitting, which directs light upwards. Wall mounted |
| Wall Light |  | Wall mounted only |
| Fixed Floor Light | No picture example – these fittings will be very rare in houses | Fixed in the floor |
| Desk Lamp |  | Plug in, multi task lamp |
| Table Lamp |  | Plug in, decorative rather than task orientated |
| Floor/Standard Lamp |  | Plug in, can be decorative or task orientated. Generally lamp is set some distance from base |
| Combination Heat/Light Unit |  | Normally found in bathrooms, may comprise of two or more heating lamps usually also with lighting lamp |
| Nightlight |  | Plug in, may be coloured, provides ambience rather than useful light |
| Rangehood | No picture example – this fitting is found in any Rangehood appliance | Normally found in kitchens, may comprise of one or more lights |
| Floodlight/External Spotlight |  | Normally outside lighting, may be directional, usually high wattage |
| Garden Light |  | Outside lighting, decorative rather than task orientated |
| Novelty Light (Ignore for lamp technology audit purposes) | No picture example – these fittings are generally plug in and not permanent | Decorative rather than task orientated, example include Fibreoptic lights, Lava lamps, Christmas lights, Fairy lights |
| Pool Lights | No picture example – normally found only in in-ground pools | Lamps mounted in a pool, under the surface of the water |
| Other |  | Give explanation |

Lighting Field Guide – Lamp Technology

What is the technology of the LIGHT SOURCE at the centre of the lamp?

Ignore the shape of the lamp’s outer envelope.

| Technology | Pictures | Notes |
| --- | --- | --- |
| Incandescent - mains voltage | no halogen capsule | No halogen capsule – large filament. Ignore the outer envelope in the picture, as this can be many shapes. If frosted envelope such that burner cannot be seen, assume incandescent – mains voltage. |
| Halogen - mains voltage | halogen capsule | Look for small halogen capsule at centre of lamp. Ignore the outer envelope in the LHS picture, as this can be many shapes including candle, reflector, etc. RHS picture is double-ended halogen (flood lights). |
| Halogen - low voltage |  | Look for small halogen capsule at centre of lamp. Lamp marked as “12V” or connected to transformer |
| Compact fluorescent - integral ballast |  | Look for fluorescent tube |
| Compact fluorescent - separate ballast |  | Note pin cap |
| Linear fluorescent |  |  |
| Circular fluorescent |  |  |
| LED |  | Typically have a cluster of diodes |
| Heat Lamp |  | Typically very large |

- 3. Lighting Field Guide – Lamp Shape

What is the SHAPE of the lamp’s outer envelope?

| Shape | Pictures | Notes |
| --- | --- | --- |
| A-shape |  | Pear, tubular and mushroom shaped |
| Fancy round |  | Spherical shape, can be small or large |
| Candle |  |  |
| Reflector - R |  | Includes MR (multi-facetted) reflector lamps |
| Reflector - PAR |  | Large diameter, thick quartz lens |
| Pilot |  | Small size |
| Double ended |  |  |
| (Halogen - low voltage) reflector |  |  |
| Capsule |  |  |
| (Compact fluorescent) stick |  |  |
| (Compact fluorescent) spiral |  |  |
| (Compact fluorescent) covered |  |  |
| (Compact fluorescent) reflector |  | Directional light source |
| (LED) reflector |  | Directional light source |
| (LED) non-reflector |  | Multi-directional light source |

### 3.1  Cap

| Cap | Pictures | Notes |
| --- | --- | --- |
| E14 |  | Small screw |
| E27 |  | Large screw |
| B15 |  | Small bayonet |
| B22 |  | Large bayonet |
| GU10 |  | Note pins have enlarged heads |

- 4. Transformer

| Transformer | Pictures | Notes |
| --- | --- | --- |
| Halogen – low voltage – magnetic |  | Approx 200+ mm long |
| Halogen – low voltage - electronic |  | Smaller and lighter than magnetic |

A joint initiative of Australian, State and Territory and New Zealand Governments

APPENDIX 4 - 2010 Residential Lighting Report

www.energyrating.gov.au